School of Computer Science & Engineering

Laboratory & Cleanroom Process -

Standard Operating Procedure

ALL FORMS SHOULD BE SUBMITTED ONE WEEK BEFORE THE PROCESS IS TO BE CARRIED OUT.

In order to remove the chip from the board it must be soaked in a solvent for a number of hours so it can be loosened without damaging the board.

Brief Description of Process:

- Administration Details:

|  | Zengbo Wang, Joshua Dunbar, Ciaran Brown |  |  |
| --- | --- | --- | --- |
| Email | z.wang@bangor.ac.uk Zengbo Wang jsd22zyy@bangor.ac.uk Joshua Dunbar crb22ljh@bangor.ac.uk Ciaran Brown |  |  |
| Room Number | 121A Excimer Laser Room |  |  |
| Supervisor | Liyang Yue, Zengbo Wang | Assessed by |  |
| Date | 15/12/25 | Date |  |
| Signature of User |  |  |  |
| Signature of Supervisor |  | Signature of Assessor |  |

Signatures in the boxes above indicate that both the user, supervisor and assessor are satisfied that the user is competent to carry out the process safely and has received and read any associated COSHH forms.

A 3rd party user is defined as someone from outside the School of Computer Science & Engineering.

By signing this form, you agree to comply with all the safety procedures currently in place in the relevant laboratory/s at the School of Computer Science & Engineering.

You also confirm that you have read and understood the relevant MSDS for all the chemicals involved in this process, and that you will adhere to all control measures on this form.

Any incidents involving spills or hazardous occurrences must be recorded in an accident or near-miss form and reported so that action can be taken to change procedures if necessary.

- Hazards:

Chemical hazards, fire risk, skin & eye irritation

(i) Identity of Hazard

Chemical, physical

(ii) Form of Hazard

10ppm (40mg/m3), 8-h TWA

(iii) Exposure limits

Users and people in the lab

(iv) Persons potentially at risk

- Level of Risk (this must be a realistic assessment based on the MSDS):

|  | Likelihood (0-5) | Severity (0-5) | L x S (0-25) | = Risk Level |
| --- | --- | --- | --- | --- |
| Inherent Risk No controls | 3 | 4 | 12 | MEDIUM |
| With Control Measures | 1 | 4 | 4 | LOW |

Note: Level of Risk follows the format used by SAFETY in their risk assessments:

LOW 0 - 7, MEDIUM 8 – 16, HIGH 17 – 25.

- Control Measures

Conduct all work in a fume hood or well-ventilated area. Work on a spill tray or absorbent surface and keep containers closed when not in use. Always wear PPE chemical-resistant gloves, goggles, and lab coats. Store flammable solvents away from heat sources and ensure no open flames or sparks are present in the workspace.

(Please highlight any additional equipment, gloves and other PPE required):

Procedure

- Ensure the PCB is fully powered off and disconnected from all equipment.

- Place the PCB into a chemically resistant container (e.g. glass or metal) that allows the board to be held at an angle, so that only the chip, its legs, and bonding adhesive are exposed to the solvent.

- Position the container on a spill tray inside a fume hood.

- Carefully decant a small volume of NMP into the container, ensuring that:

  - The NMP covers only the broken chip and surrounding adhesive, and

  - The remainder of the PCB is kept above the solvent level.

- Leave the PCB under the fume hood for up to 2 hours to allow the NMP to soften the adhesive.

- After soaking, remove the PCB from the container and place it on an absorbent, solvent-resistant surface.

- Apply gentle pressure to the chip using ESD-safe tools to check whether it has loosened.

- If the chip remains bonded, return the PCB to the container and repeat the soaking step as required.

- Once the chip has been removed, immediately rinse the treated area with deionised water to remove residual NMP and minimise surface degradation or oxidation.

- Dry the PCB on a low-temperature hot plate or using air/nitrogen flow inside the fume hood.

- Dispose of all NMP, wipes, gloves, and contaminated materials as hazardous solvent waste in accordance with local regulations.

Standard Operating Procedure:

- Actions in the case of an accident or emergency associated with the procedure:

1. Chemical Spill

Action:

- Evacuate the area if the spill releases significant solvent vapours or is large.

- Alert others in the vicinity and restrict access to the affected area.

- Use appropriate spill containment materials (e.g., absorbent pads or spill kits).

- Wear proper PPE (gloves, goggles, and respirator if necessary).

- Dispose of spilled material and contaminated items according to the Safety Data Sheet (SDS).

Prevention: Work on a spill tray or absorbent surface and keep containers closed when not in use.

2. Skin or Eye Contact

Action:

- Skin Contact: Wash the affected area with soap and water for at least 15 minutes. Remove contaminated clothing.

- Eye Contact: Flush eyes with water using an eye wash station for at least 15 minutes. Seek medical attention immediately.

Prevention: Always wear chemical-resistant gloves, goggles, and lab coats.

3. Inhalation of Vapours

Action:

- Move the affected person to fresh air immediately.

- Seek medical attention if symptoms like dizziness, nausea, or respiratory distress occur.

- Ensure proper ventilation in the workspace.

Prevention: Conduct all work in a fume hood or well-ventilated area.

4. Fire or Explosion

Action:

- Evacuate the area, close the door to the laboratory behind you, and activate the fire alarm.

- Shut down heating devices (e.g., hot plates or ovens) if safe to do so.

Prevention: Store flammable solvents away from heat sources and ensure no open flames or sparks are present in the workspace.

5. Chemical Exposure to Clothing Action: - Remove contaminated clothing immediately and isolate it in a chemical waste bag. - Wash exposed skin thoroughly with soap and water. Prevention: Wear a lab coat or apron to protect clothing.