COSHH RISK ASSESSMENT FORM (Chemicals / Materials)

| Name: Zengbo Wang | Contact Details: z.wang@bangor.ac.uk | Date: 15/12/2025 |
| --- | --- | --- |
| College: The College of Environmental Sciences and Engineering | School: School of Computer Science and Engineering |  |
| Personnel Affected: Zengbo Wang Joshua Dunbar Ciaran Brown | Academic / Supervisor Responsible: Zengbo Wang |  |
|  | Contact Details: z.wang@bangor.ac.uk |  |
|  | You are confirming chemicals are not on the prescribed list. http://www.bangor.ac.uk/hss/inflink/statutory.php.en |  |

| PROCEDURE TITLE: 1-Methyl-2-pyrrolidone purchasing |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| List Hazardous Chemicals Used | CAS No. | Quantity Used | Quantity Handled | Conc. | Workplace Exp. Limits | Hazards |
| N-Methyl-2-pyrrolidone (NMP) | 872-50-4 | 100ml | 1L |  | 10ppm 40mg/m3 8-h TWA | H315 Causes skin irritation H319 Causes serious eye irritation H335 May cause respiratory irritation H360D May damage the unborn child |

| ALWAYS CHECK BEFORE STARTING ANY EXPERIMENT |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A less hazardous substance can be used | YES | NO |  |  |  | The substance is a known carcinogen |  | YES |  | NO |
| The substance is a known mutagen | YES | NO |  |  |  | The substance is a known teratogen |  | YES |  | NO |
| The procedure can enhance hazards | YES | NO |  |  |  | The procedure can create harmful bi-products |  | YES |  | NO |
| The procedure can cause an exothermic reaction | YES | NO |  |  |  | The procedure can cause a build up of pressure |  | YES |  | NO |
| Work to be carried out (circle) | Open Bench | Fumehood Required |  |  |  | If Fumehood must be used is Fire Trace needed |  | YES |  | NO |

| EXPERIMENTAL PROCEDURE: |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Ensure the PCB is fully powered off and disconnected from all equipment. - Place the PCB into a chemically resistant container (e.g. glass or metal) that allows the board to be held at an angle, so that only the chip, its legs, and bonding adhesive are exposed to the solvent. - Position the container on a spill tray inside a fume hood. - Carefully decant a small volume of NMP into the container, ensuring that: - The NMP covers only the broken chip and surrounding adhesive, and - The remainder of the PCB is kept above the solvent level. - Leave the PCB under the fume hood for up to 2 hours to allow the NMP to soften the adhesive. - After soaking, remove the PCB from the container and place it on an absorbent, solvent-resistant surface. - Apply gentle pressure to the chip using ESD-safe tools to check whether it has loosened. - If the chip remains bonded, return the PCB to the container and repeat the soaking step as required. - Once the chip has been removed, immediately rinse the treated area with deionised water to remove residual NMP and minimise surface degradation or oxidation. - Dry the PCB on a low-temperature hot plate or using air/nitrogen flow inside the fume hood. - Dispose of all NMP, wipes, gloves, and contaminated materials as hazardous solvent waste in accordance with local regulations. |  |  |  |  |
| CONTROLS: |  |  |  |  |
| Operational: | PPE Required: (delete / detail as needed) |  |  |  |
|  | Gloves | Nitrile | Latex | Other |
|  | Face shield | Non UV resistant | UV resistant | Other |
|  | Eye Prot. | Safety glasses | Safety goggles | UV resistant |
|  | Footwear | Detail: Closed shoes |  |  |
|  | Mask | Detail: Face mask |  |  |

| REQUIRED CONTROL(s) CHECKS: |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reaction Temp. | Y | N | Reaction Time | Y | N | PPE Type / Integrity | Y | N | Fumehood Operation | Y | N |

| ACTION TO TAKE AT THE END OF THE EXPERIMENT |  |  |  |
| --- | --- | --- | --- |
| MATERIALS TO BE RETAINED: (if applicable detail) |  |  |  |
| Materials: NMP (872-50-4) | Specific Hazards: H315 Causes skin irritation H319 Causes serious eye irritation H335 May cause respiratory irritation H360D May damage the unborn child | Cleaning Required: Ensure exterior of container is clean and free from spills. Wipe bottle with lint-free tissue if contaminated. Clean any small spills immediately using appropriate spill absorbent | Storage / Labelling: Keep container tightly closed when not in use. Store away from heat, sparks, and ignition sources.in lab 121A on the first floor, Dean Street. |
| DISPOSAL OF WASTE CHEMICALS: (if applicable detail) |  |  |  |
| Chemicals AND Quantity: Waste NMP | Specific Hazards: Hazards as discussed above | Containers Required: (check correct number & type eg ventilated lid available) Aqueous waste bottle | Storage / Labelling: Storage in a waste bottle in fume cupboard. |

| EMERGENCY PROCEDURES |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FIRST AID: (tick as appropriate) |  |  |  |  |  |  |  |  |
| Chemical | Ingested |  | Inhalation |  | Eye Contact |  | Skin Contact |  |
|  | Induce vomiting | Medical attention | Fresh air | Medical attention | Rinse lots of water | Medical attention | Rinse lots of water | Medical attention |
| NMP (872-50-4) |  |  |  |  |  |  |  |  |
| Additional First Aid action to take if applicable: N/A |  |  |  |  |  |  |  |  |

SPILLAGE:

Large Scale Spill Procedure: Prevent further spillage or leakage if safe to do so. Contain spill and absorb using absorbent material (e.g. spill pads or granules). Collect absorbed material into a suitable waste container. Dispose of waste according to solvent disposal guidelines. Ensure adequate ventilation.

Small Scale Spill Procedure: Prevent further spillage or leakage, contain spillage and then collect with an absorbant material. Then dispose of materials and liquid according to disposal guidelines of the specific solvent spillage.

| FIRE AND EXPLOSION RISK: (delete / detail as appropriate) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fire Risk? | Y | N | Explosion Risk? | Y | N | Toxic Fumes Risk? |  | Y | N |
| If ‘Y’ detail what you are doing to control the risk(s): Avoid open flames and sparks in the lab while working with the NMP. Do all work under the fume hood to avoid exposure or build up of vapours. Use minimum quantities required inside fume hood. Keep containers tightly closed when not in use. |  |  |  |  |  |  |  |  |  |
| Detail the action to take in a fire: No action shall be taken involving any personal risk or without suitable training. Evacuate the area and call emergency services/ security. |  |  |  |  |  |  | List specific fire extinguishers: (if needed) foam fire extinguisher |  |  |