COSHH ASSESSMENT FORM (Non-Biological)

| COLLEGE / DEPARTMENT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Personnel Affected: Cleanroom users |  |  |  |  |  |  |  |  |  |  | Academic / Supervisor Responsible: Dr Zengbo Wang |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| **The CAS # MUST be filled in *** |  |  |  |  |  |  |  |  |  |  | You are confirming that these chemicals are not on the proscribed list. See http://www.bangor.ac.uk/hss/inflink/statutory.php.en |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Academic Internal Phone details: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PROCEDURE TITLE: PMMA Polymerization |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| List Hazardous Chemicals Used |  | CAS # |  |  |  |  | Quantity Used |  | Quantity Handled |  |  | Conc. |  |  |  |  |  | Workplace Exp. Limits |  |  | Hazards |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Glutaric dialdehyde solution |  | 111-30-8 |  |  |  |  | <10ml |  | <500ml |  |  |  |  |  |  |  |  | Hood in room 121 |  |  | - Acute toxicity, (Category 4) H302: Harmful if swallowed. - Acute toxicity, (Category 3) H331: Toxic if inhaled. - Skin corrosion, (Sub-category 1B) H314: Causes severe skin burns and eye damage. - Serious eye damage, (Category 1) H318: Causes serious eye damage. - Respiratory sensitization, (Category 1) H334: May cause allergy or asthma symptoms or breathing difficulties if inhaled. - Skin sensitization, (Category 1) H317: May cause an allergic skin reaction. - Specific target organ toxicity -single exposure, (Category 3), Respiratory system H335: May cause respiratory irritation. - Short-term (acute) aquatic hazard, (Category 1) H400: Very toxic to aquatic life. - Long-term (chronic) aquatic hazard, (Category 2) - H411: Toxic to aquatic life with long lasting effects. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ALWAYS CHECK BEFORE STARTING ANY EXPERIMENT IF… |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A less hazardous substance can be used |  |  |  |  |  | The substance is a known carcinogen |  |  |  |  |  |  |  |  |  |  | The substance is a known mutagen |  |  |  |  |  |  |  |  | The substance is a known teratogen |  |  |  |  |  |  |  |  |  |  |
| Experimental Procedure: In fume hood. Mix Base and algae suspension with 100:1 ratio. Take small volume of algae suspension and drop on silicon chip surface. Measure the sample with microscope after full drying. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONTROLS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operational: Work only in fume hood. Wear USA made N95 mask, coverall, nitrile gloves and safety goggles. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | PPE Required: (delete, detail as appropriate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Gloves |  |  |  |  | Nitrile |  |  |  | Other |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Face shield |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Eye Prot. |  |  |  |  |  | Safety goggles |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Footwear |  |  |  |  | Detail: |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Mask |  |  |  |  | Detail: USA made N95 mask |  |  |  |  |  |  |  |  |  |  |
| MONITORING REQUIRED: (delete and provide details as appropriate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reaction Temp. |  |  |  | Y | Reaction Time |  |  |  |  |  |  |  | Y |  | Integrity of PPE |  |  |  |  |  |  | Y |  |  |  | Fume Cabinet Op. |  |  |  |  | Y |  |  |  |  |
| EMERGENCY PROCEDURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FIRST AID: (tick as appropriate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chemical |  |  | Ingested |  |  |  |  |  |  | Inhalation |  |  |  |  |  |  |  |  | Eye Contact |  |  |  |  |  |  |  | Skin Contact |  |  |  |  |  |  |  |  |
|  |  |  | Induce vomiting |  |  |  |  | Medical attention |  | Fresh air |  |  |  |  |  | Medical attention |  |  | Rinse lots of water |  |  |  | Medical attention |  |  |  | Rinse lots of water | Medical attention |  |  |  |  |  |  |  |
| Glutaric dialdehyde solution |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Additional First Aid action to take if applicable: Eye Contact: Check for and remove any contact lenses. In case of contact, immediately flush eyes with plenty of water for at least 15 minutes. Get medical attention. Skin Contact: Wash with soap and water. Cover the irritated skin with an emollient. Get medical attention if irritation develops. Serious Skin Contact: Not available. Inhalation: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical attention. Serious Inhalation: Not available. Ingestion: Do NOT induce vomiting unless directed to do so by medical personnel. Never give anything by mouth to an unconscious person. Loosen tight clothing such as a collar, tie, belt or waistband. Get medical attention if symptoms appear.In case of contact with eyes, flush with copious amounts of water for at least 15 minutes. Assure adequate flushing by separating the eyelids with fingers. Call a physician. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPILLAGE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FIRE AND EXPLOSION: (delete and detail as appropriate) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Is there a risk of fire? |  |  |  |  |  |  |  |  |  |  |  |  |  | Y |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Is there a risk of explosion? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Is there a risk of toxic fumes from fire? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | What are you doing to control the risk? Keep away from heat sources. Keep away from sources of ignition. Ground all equipment containing material. Do not ingest. Do not breathe gas/fumes/ vapor/spray. Avoid contact with eyes. Wear suitable protective clothing. If ingested, seek medical advice immediately and show the container or the label. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Small scale: : Absorb with an inert material and put the spilled material in an appropriate waste disposal. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Detail the action to take in a fire: Self-contained breathing apparatus and protective clothing should be worn in fighting large fires involving chemicals. Determine the need to evacuate or isolate the area according to your local emergency plan. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Detail suitable fire extinguishers: Water spray. Carbon dioxide (CO2). Dry chemical. Foam. Water can be used to cool fire exposed containers. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ACTION TO TAKE AT THE END OF THE EXPERIMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Materials retained for future use? - detail hazards, special storage and labelling requirements Precautions: Keep away from heat. Keep away from sources of ignition. Ground all equipment containing material. Do not ingest. Do not breathe gas/fumes/ vapor/spray. Avoid contact with eyes. Wear suitable protective clothing. If ingested, seek medical advice immediately and show the container or the label. Storage: Tightly closed. Keep in a well-ventilated place. Keep locked up or in an area accessible only to qualified or authorized persons. Store under inert gas. Air sensitive |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Disposal - detail what action needs to be taken Waste must be disposed of in accordance with federal, state and local environmental control regulations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name : |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: Academic Supervisor to sign here. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |